# Page 1 of 4

# MATERIAL SAFETY DATA SHEET

### PRODUCT :  PETROLEUM JELLY

Fire

## HAZARD RATINGS

N 4 – Extreme						          2

F 3 – High

P 2 – Moderate                                      Toxicity               0           0        Reactivity

A 1 – Slight

0 – Insignificant

Special

=========================================================

### CHEMICAL AND PHYSICAL PROPERTIES – SECTION 1

=========================================================

Formula :  not applicable

Chemical Name :  Petroleum Jelly.

Hazardous Decomposition Products : carbon monoxide and carbon dioxide from burning

Incompatibility (keep away from):  strong oxidizers such as hydrogen perodixe, bromine and chromic acid.

Toxic and Hazardous Ingredients: none

Form:  Semi- solid, translucent soft mass	            Odor : none

Appearance:  Semi-solid					Color : white and Yellow

Specific Gravity (water=1): less than 1

Boiling Point: no data available

Melting Point:  48 to 52C (11 8.4 to 125.6 F

Solubility in Water (by weight %): 0

Volatile (by weight %): Max 0.5%

Page 2 of  4

Evaporation Rate: not applicable

Vapor Pressure (mm Hg at 20C): not applicable

Vapor Density (air=1): not applicable

PH (as is): not applicable

Stability: Product is stable under normal conditions

Viscosity SUS at 100F: not applicable

=========================================================

### FIRE AND EXPLOSION DATA – SECTION II

=========================================================

Special Fire Fighting Proceedures:  Wear self-contained breathing apparatus.  Water spray is an unsuitable extingushing agent. (See extinguishing Agents)

Unusual Fire and Explosion Hazards: none

Flashpoint:  (Method Used – ASTM D-92) greater than 198.88C (390F)

Flammability limits %: no data available

Extinguishing agents:  Drychemical or Waterfog or CO² or Foam or Sand/Earth.  Closed containers exposed to fire may be cooled with water.

===================================================

HEALTH HAZARD DATA – SECTION III

=========================================================

Permissible concentrations (air): no data available

Chronic effects of overexposure: no data available

Acute toxicological properties:  no data available

Page 3 of 4

Emergency First Aid Procedures:

Eyes:	For molten product only, immediately flush with large quantities of water for at least 15 minutes and call a physician.

Skin Contact:	If burned by contact with hot material, cool burned skin area as quickly as   possible by immersing in cold water, or applying cold water.  Call a physician.

Inhalation:	not applicable

If Swallowed:	Contact physician immediately

=========================================================

### SPECIAL PROTECTION INFORMATION- SECTION IV

=========================================================

Ventilation Type Required (Local, mechanical, special): none

Respiratory Protection (Specify type): none

Protective Gloves: heat protective impervious gloves when handling molten product.

Eye Protection:  Chemical safety goggles and if handled hot, full face shield

Other Protective Equipment: coveralls when handling  molten products

=========================================================

### HANDLING OF SPILLS OR LEAK-SECTION V

=========================================================

Procedures for Clean-up:  Shut off leak and dike up large spills.   After cooling and solidification, scrape and/or shovel up material.  Finaly, clean area with an oil absorbent material.

Waste Disposal:  Dispose of in accordance with all applicable Federal, State and local regulations.

Page 4 of 4

=========================================================

### SPECIAL PRECAUTIONS- SECTION VI

=========================================================

Precautions to be taken in handling and storage:  Keep away from heat and flames.

Do not store near strong oxidizing agents.

For  Petroleum Jelly shipped in Bulk:  Product will normally be maintained in a heated liquid form.

Excise suitable precautions to avoid thermal burns.  Avoid breathing mist or fumes/vapours from hot material.

Maximum Storage Temperature:  80 C  (176 F )

=========================================================

#### TRANSPORTATION DATE – SECTION VII

=========================================================

D.O.T.:  Not Regulated

Reportable Quantity: not applicable

Freight Classification:  Petroleum Wax, N.O.I.B.N.

Special Transportation Notes: none

=========================================================

# LABEL INFORMATION

===================================================

WARNING:  When handling  molten material, wear protective clothing and gloves, wear a face shield over splash goggles.  Avoid all ignition sources such as flames and sparks.

Hazard warning label not required under OSHA Hazard Communication Standard 29 CFR 1910.1200

SPILLS OR LEAKS:  Shut off leak and dike up large spills.  After cooling and solidification, scrape and/or shovel up material.  Finally, clean area with an oil absorbent material.  Dispose of in compliance with federal, state and local regulations.

FOR MANUFACTURING USE ONLY.